https://youtu.be/eVlavqMN9vI?si=dfHLB0LSlKb90LJJ

do you know UniFy

###### Copilot said:

Copilot

Yes — “UniFy” can mean a few different things, depending on the context. Here are the most common ones people usually refer to:

### 1. UniFi (by Ubiquiti) – most common

Often spelled UniFi (not UniFy), this is a very popular networking platform:

- Wi‑Fi access points

- Network switches & gateways

- Centralized network management (UniFi Controller)
Used a lot in companies, campuses, hotels, and ISPs.

👉 If you’re talking about Wi‑Fi, routers, or network equipment, this is probably it.

### 2. UniFy / Unify as a software or platform name

Some companies use Unify / UniFy for:

- ERP or business management systems

- HR or payroll platforms

- Fintech or digital payment solutions

- University or student platforms (sometimes locally developed)

These are usually regional or company‑specific, especially in Africa.

### 3. Unify (formerly Siemens Unify)

Unify was also a communications and collaboration company (telephony, VoIP, enterprise communications), later associated with Atos.

To help me give you the right answer, tell me:

- Are you asking about network/Wi‑Fi, software, fintech, or something else?

- Where did you hear about UniFy (work, school, internet, company name)?

I’ll tailor the explanation exactly to what you need.

##### You said:

I am talking about the first one

###### Copilot said:

Copilot

Got it 👍 — you mean UniFi by Ubiquiti.

Here’s a clear, practical overview of UniFi and how it works.

## What is UniFi (Ubiquiti UniFi)?

UniFi is an enterprise-grade networking ecosystem made by Ubiquiti, designed to manage:

- Wi‑Fi networks

- Switching

- Routing / security

- Network monitoring

All managed from one central interface (the UniFi Controller).

It’s widely used in:

- Companies & offices

- Universities & schools

- Hotels

- ISPs

- Large homes (advanced users)

## Main UniFi Components

### 1. UniFi Access Points (APs)

These provide Wi‑Fi.

- U6 Lite, U6 LR, U6 Pro, U7 (newer)

- Indoor & outdoor models

- Seamless roaming (users move without disconnecting)

✅ Best known UniFi product

### 2. UniFi Switches

Network switches with centralized control.

- PoE switches (power APs & cameras)

- Managed VLANs

- Traffic monitoring

Examples:

- UniFi Switch Lite

- UniFi Switch Pro

### 3. UniFi Gateways / Routers

Handle internet connection & security.

- UniFi Dream Router (UDR)

- UniFi Dream Machine (UDM, UDM Pro, SE)

- Advanced firewall, VPN, IDS/IPS

### 4. UniFi Controller (Network Application)

The brain of UniFi.

You can run it on:

- UniFi Cloud Key

- UniFi Dream Machine

- A PC / Linux server

- Cloud hosting

From ONE dashboard you can:

- Configure Wi‑Fi & switches

- Monitor users

- See bandwidth usage

- Apply security rules

- Update devices

### 5. Other UniFi Systems (optional)

UniFi is more than networking:

- UniFi Protect – Security cameras (CCTV)

- UniFi Access – Door access & badges

- UniFi Talk – VoIP phones

All managed in the same ecosystem.

## Why UniFi Is So Popular

✅ Centralized management
✅ Enterprise features at lower cost
✅ Scalable (small office → large campus)
✅ Very stable Wi‑Fi
✅ No license fees (huge advantage)

## Typical UniFi Network Example

Internet

│

UniFi Gateway (UDM)

│

UniFi Switch (PoE)

│

UniFi Access Points

↓

Users (phones, laptops, cameras)

Everything is configured from one dashboard.

## UniFi vs Regular Routers

| Feature | Normal Router | UniFi |
| --- | --- | --- |
| Central control | ❌ | ✅ |
| Multiple APs | Limited | ✅ Excellent |
| Enterprise security | ❌ | ✅ |
| Scalability | Low | Very high |

## Is UniFi Good for You?

UniFi is perfect if you:

- Manage many users

- Need stable Wi‑Fi

- Want professional networking skills

- Work in IT / networking

If you just want Wi‑Fi for 1–2 phones, it’s overkill.

If you want, I can:

- Explain UniFi for beginners

- Show how to set up UniFi step by step

- Help you design a UniFi network for your company

- Explain UniFi vs MikroTik vs TP‑Link Omada

Just tell me what you’re aiming to do 👍

##### You said:

I would like to use  Unifi in my hotel network for my client but i've never used unifi before what shall I do in this case

###### Copilot said:

Copilot

Great question — and you’re doing the right thing by asking before deploying it for a client. UniFi is an excellent choice for hotels, even if you’ve never used it before.

Below is a practical, step‑by‑step path you can follow to successfully deploy UniFi in a hotel network, even as a first‑timer.

I’ll keep it realistic, field‑tested, and client‑safe.

# ✅ Big Picture: What You Should Do

Your goal
Build a stable, secure, easy‑to‑manage hotel Wi‑Fi that:

- Works in all rooms

- Separates guests from hotel staff

- Allows guest portal (voucher / password)

- Doesn’t require constant maintenance

UniFi is PERFECT for this.

# STEP 1 — Learn Just Enough UniFi (Before Touching Client Network)

You do NOT need to become an expert.

Focus only on these concepts:

### ✅ Core things to understand

- UniFi Controller (Network Application)

- Adoption (devices must be “adopted” into controller)

- Wi‑Fi Networks (SSIDs)

- VLANs (Guest vs Staff)

- Guest Portal

If you understand just these → you can deploy a professional hotel network.

👉 Tip: UniFi uses very visual dashboards, so beginners learn fast.

# STEP 2 — Choose the RIGHT UniFi Hardware for a Hotel

### 🔹 Recommended Simple & Safe Setup

For a hotel, avoid complicated gear.

### ✅ Best core device

UniFi Dream Machine Pro (UDM‑Pro) or UDM‑SE

- Router + Firewall

- Runs UniFi Controller

- Supports Guest Portal

- VPN + Security

✅ This avoids installing software on a PC.

### ✅ Switches

- Use UniFi PoE switches

- One switch per floor (best practice)

Example:

- UniFi Switch Lite 16 PoE

- UniFi Switch Pro 24 PoE (bigger hotels)

✅ PoE powers Access Points — no power adapters needed.

### ✅ Access Points (VERY important)

Hotels need many APs, not powerful ones.

Best models:

- U6 Lite → guest rooms

- U6 LR → corridors / lobby

- U6 Mesh → outdoor areas

📌 Do not use 1 AP for many rooms — Wi‑Fi quality will be bad.

# STEP 3 — Design the Network (Before Installation)

### ✅ Minimum VLAN design

You MUST separate traffic.

| Network | Purpose |
| --- | --- |
| VLAN 10 | Hotel Staff (front desk, admin) |
| VLAN 20 | Guests |
| VLAN 30 | Devices (cameras, POS, printers) |

✅ UniFi makes this easy in the UI.

### ✅ Wi‑Fi SSIDs

Create separate Wi‑Fi networks:

- Hotel‑Guest

  - Guest Portal (voucher or password)

  - Internet only

  - No access to other devices

- Hotel‑Staff

  - Strong password

  - Full access to internal services

# STEP 4 — Install UniFi Controller & Adopt Devices

### ✅ If using UDM

- Plug it in

- Access via browser: https://unifi.ui.com

- Create UniFi account

- Controller starts automatically

### ✅ Adopt devices

When APs & Switches are connected:

- They appear as “Pending Adoption”

- Click Adopt

- Wait for provisioning

✅ This is VERY beginner‑friendly.

# STEP 5 — Configure Guest Wi‑Fi (Very Important for Hotels)

### ✅ Enable Guest Control

In UniFi:

Settings → WiFi → Add WiFi

→ Enable Guest Policy

### ✅ Guest Portal options

Choose what fits the hotel:

- Simple password (same for all guests)

- Voucher codes

- Time‑limited access

✅ UniFi built‑in portal is OK for most hotels.

### ✅ Bandwidth limits (recommended)

Prevent one room from killing the Wi‑Fi:

- Download limit per client

- Upload limit per client

# STEP 6 — Security & Stability Essentials

### ✅ MUST‑DO settings

- Enable Fast Roaming

- Enable Band Steering

- Enable Client Isolation on Guest WLAN

- Disable UPnP on guest VLAN

- Use WPA2/WPA3

### ✅ Firewall rules (basic)

- Guest → Internet ✅

- Guest → Internal networks ❌

- Staff → Internal ✅

UniFi has templates — you don’t need to write rules manually.

# STEP 7 — Test Like a Guest

Before handing over: ✅ Walk with phone from room to corridor
✅ Test portal login
✅ Test speed limits
✅ Try connecting 10+ devices
✅ Restart one AP and see recovery

If this works → client will be happy.

# STEP 8 — Learn Safely (NO RISK TO CLIENT)

### ✅ Best learning method

- Build UniFi in a lab

  - 1 AP

  - 1 UDM

  - Home internet

- Practice:

  - Create SSIDs

  - Enable guest portal

  - Play with VLANs

Only after → deploy to hotel.

# What NOT To Do (Important Warnings)

❌ Don’t mix UniFi with random switches
❌ Don’t use ISP router as main router
❌ Don’t place APs inside rooms only
❌ Don’t leave guest & staff on same network

# If You Want, I Can Help You Further

I can: ✅ Design the AP placement for your hotel
✅ Create a ready‑to‑use UniFi configuration checklist
✅ Help you choose exact models based on hotel size
✅ Explain UniFi as if you’ve never touched it before

To continue, tell me:

- How many rooms?

- Number of floors?

- ISP speed?

- Budget level (low / medium / high)

We’ll design it properly and safely for your client.

##### You said:

here is the near config of the hotel: 18 bedrooms at entry point we got a story building that has one floor but below it a reception and nightclub is beside the reception but at first floor we got the restaurant where people should pay for connecting to the network but folk will see themselves granted 1 hour of connection before that goes off and they have to buy for it to be able to connect again.

###### Copilot said:

Copilot

Perfect — thanks, this description is clear enough to design the full UniFi solution.
I’ll give you a realistic, deployable UniFi design specifically for your hotel layout and business rules, without assuming prior UniFi experience.

# ✅ Hotel UniFi Network Design (Your Case)

## 🏨 Hotel Summary (from your description)

- 18 bedrooms

- Entry building with 1 floor

- Ground level:

  - Reception

  - Nightclub (next to reception)

- First floor:

  - Restaurant

- Business rule for restaurant Wi‑Fi:

  - Everyone gets 1 hour free

  - After 1 hour → access blocked

  - User must pay to reconnect

✔️ UniFi can do this natively (no external system required).

# ✅ Recommended UniFi Architecture (Simple & Professional)

## 🔹 Core Device (Mandatory)

### UniFi Dream Machine SE (or UDM‑Pro)

Why:

- Acts as router + firewall + controller

- Has built‑in captive portal

- Handles time‑based guest access

- Stable for hotels

✅ Do NOT use ISP router as main router
✅ Put ISP router in bridge mode

## 🔹 Switching

### 1 PoE Switch

Recommended:

- UniFi Switch Lite 16 PoE or

- UniFi Switch Pro 24 PoE (if expansion planned)

Purpose:

- Powers all APs

- Centralized management

## 🔹 Access Points (Correct Quantity Matters)

### ✅ AP Placement (VERY IMPORTANT)

Avoid “strong single AP” thinking.

| Area | AP Model | Quantity |
| --- | --- | --- |
| Guest rooms (corridors) | U6 Lite | 3 |
| Reception + Lobby | U6 LR | 1 |
| Nightclub | U6 LR | 1 |
| Restaurant (1st floor) | U6 LR | 1 |

✅ Total APs: 6

📌 Reason:

- Good coverage

- Prevent overload

- Seamless roaming

# ✅ Network Segmentation (Critical for Hotels)

You will create 3 networks (VLANs):

| VLAN | Name | Used For |
| --- | --- | --- |
| VLAN 10 | STAFF | Reception, admin |
| VLAN 20 | GUEST | Bedrooms |
| VLAN 30 | RESTAURANT | Paid Wi‑Fi |

UniFi makes this UI‑based, no manual config.

# ✅ Wi‑Fi (SSIDs) Design

### 1️⃣ Hotel‑Guest

- Used in bedrooms

- No guest portal

- Simple password (rotated monthly)

- Bandwidth limited per device

- Client isolation ON

### 2️⃣ Hotel‑Staff

- Hidden SSID (optional)

- Strong password

- Full internal access

### 3️⃣ Restaurant‑WiFi ⭐ (MOST IMPORTANT)

This SSID handles: ✅ 1 hour free ✅ Pay after expiry

# ✅ How to Implement the 1‑Hour Free Wi‑Fi (UniFi Native)

## 🔹 Step 1: Enable Guest Policy

Settings → WiFi → Create WiFi

→ Name: Restaurant‑WiFi

→ Enable Guest Policies

→ Assign VLAN 30

## 🔹 Step 2: Configure Guest Portal

Settings → Guest Control

Configure:

- ✅ Captive Portal: ON

- ✅ Authentication: Simple click-through OR voucher

- ✅ Session Timeout: 60 minutes

- ✅ Idle Timeout: 10–15 minutes

- ✅ Landing Page: Custom text

- “You have 1 hour free internet. Please see staff to continue after expiry.”

✅ After 1 hour → device is disconnected automatically.

## 🔹 Step 3: Payment Flow (Simple & Practical)

UniFi does NOT process money.
This is how hotels do it professionally:

### ✅ Recommended Method (Easy & Reliable)

- Guest gets 1 hour free

- Internet stops

- Guest pays at reception or restaurant cashier

- Staff gives:

  - Voucher code (generated in UniFi) OR

  - Static paid password (time-limited)

✅ UniFi voucher system:

- 1 hour

- 3 hours

- 24 hours

- 7 days

📌 No external billing platform needed initially.

## 🔹 Voucher Configuration

Settings → Guest Control → Vouchers

Create:

- 1‑Hour Paid

- 6‑Hour Paid

- Daily

- Weekly

Reception staff can generate vouchers in 2 clicks.

# ✅ Nightclub Wi‑Fi Strategy

Two choices (you pick):

### 🔹 Option A (Recommended)

- Same Restaurant‑WiFi

- Free 30–60 minutes

- Then voucher

### 🔹 Option B

- Separate SSID: Nightclub‑WiFi

- Shorter session

- Strong bandwidth limits

# ✅ Security Essentials (Must Enable)

✅ Guest Isolation
✅ Block Guest → Internal traffic
✅ Band Steering ON
✅ Fast Roaming ON
✅ Limit per‑client speed (e.g. 5–10 Mbps)
✅ Disable peer‑to‑peer sharing

UniFi has toggles for all of this.

# ✅ Final Logical Topology

ISP ONT

↓

UDM‑SE (Controller + Firewall)

↓

UniFi PoE Switch

↓

Access Points

├── Rooms

├── Lobby

├── Nightclub

└── Restaurant

# ✅ What I STRONGLY Recommend Next

Before installing at client site:

### ✅ Build a Mini Lab

- UDM

- 1 AP

- Simulate:

  - 1‑hour cutoff

  - Voucher login

  - Reconnection

✅ This avoids embarrassment at deployment.

# ✅ I Can Take You Further (Step‑by‑Step)

If you want, next I can:

- Give exact UniFi menu paths (click by click)

- Design AP placement drawing

- Create voucher policy templates

- Prepare a handover checklist for client

- Help you write Wi‑Fi usage terms

Just tell me 👍
Do you want:

- Step‑by‑step UniFi configuration

- Hardware shopping list

- Guest portal wording

- Deployment checklist

You’re VERY close to a solid professional deployment.

Guess what in each bedroom client or guest must be able to connect their phones and TV and have VoIP calls.

Create a ready‑to‑use UniFi configuration checklist.

Help you choose exact models based on hotel size.

How much these Unifi equipments.

Which is it belongs to(Unifi) ?.

Do you get youtube video links for these configurations of Unifi.

You must know that the hotel owner bought router(Internet Access) with 30Mbps, so he would like to base his entire hotel network on it.

How could I learn it safely how having practical tools according everything you've said here.

###### Copilot said:

Copilot

Excellent — this is a real hotel‑grade requirement, and UniFi can do it.
Below is a complete, ready‑to‑use response covering configuration, models, prices, learning path, VoIP & TVs, bandwidth with 30 Mbps, and safe practice.

# 1️⃣ Bedroom Requirements: Phones, Smart TVs & VoIP

### ✅ What each bedroom needs

- 2–4 phones

- 1 Smart TV (YouTube / Netflix via guest login)

- VoIP calls (WhatsApp, SIP apps, Teams)

✅ UniFi fully supports this, as long as you configure:

- 5 GHz Wi‑Fi coverage

- Client isolation

- Per‑client speed limits

- Multicast / mDNS for TVs (optional)

📌 Smart TVs connect like phones — no cable required
📌 VoIP works well if latency is controlled (bandwidth shaping)

# 2️⃣ READY‑TO‑USE UniFi CONFIGURATION CHECKLIST ✅

Use this exact order during deployment.

## ✅ A. Hardware Setup

☐ ISP Router → Bridge mode
☐ UDM‑SE connected as main router
☐ UniFi PoE switch connected to UDM
☐ All APs powered via PoE

## ✅ B. Core Networks (VLANs)

Create in Settings → Networks

| Network | VLAN | Purpose |
| --- | --- | --- |
| STAFF | 10 | Reception, admin |
| GUEST‑ROOMS | 20 | Bedrooms |
| RESTAURANT | 30 | Paid / timed Wi‑Fi |

☐ Enable Network Isolation for VLAN 20 & 30

## ✅ C. Wi‑Fi (SSIDs)

### Hotel‑Rooms

- Security: WPA2/WPA3

- VLAN: 20

- Client isolation: ON

- Bandwidth: 3 Mbps down / 1 Mbps up

- APs: Bedroom APs only

### Restaurant‑WiFi

- Open or WPA2

- VLAN: 30

- Guest policies: ON

- Captive Portal: ON

- Session time: 60 minutes

- After expiry: voucher required

### Hotel‑Staff

- VLAN: 10

- Strong password

- No bandwidth limit

## ✅ D. Guest Portal (1‑Hour Free Rule)

Configure in Settings → Hotspot / Guest Control

✔ Session timeout: 60 minutes
✔ Idle timeout: 10–15 min
✔ Landing text:

“You have 1 hour free internet. Please contact reception to continue.”

✔ Enable Vouchers
✔ Create vouchers: 1h / 6h / 24h

📌 UniFi native vouchers — no external system

## ✅ E. Bandwidth Control (VERY IMPORTANT for 30 Mbps)

Create User Groups

| Group | Download | Upload |
| --- | --- | --- |
| Guest‑Room | 3 Mbps | 1 Mbps |
| Restaurant | 5 Mbps | 2 Mbps |
| Staff | Unlimited | Unlimited |

📌 This protects VoIP and fairness

## ✅ F. Wi‑Fi Optimization

☐ Enable Band Steering
☐ Enable Fast Roaming
☐ Enable Proxy ARP
☐ Prefer 5 GHz
☐ Disable legacy Wi‑Fi rates

# 3️⃣ Exact UniFi Models (Correct for 18‑Room Hotel)

### ✅ Ownership

UniFi belongs to Ubiquiti Inc., an American networking companywikipedia

## ✅ Core Devices

### 🔹 UniFi Dream Machine SE (UDM‑SE)

- Router + Firewall + Controller

- Guest portal & vouchers

- VoIP friendly

- Price: ~USD 499ui

### 🔹 UniFi Switch Lite 16 PoE

- Powers APs

- Silent (fanless)

- Price: ~USD 199ui

## ✅ Access Points

### 🔹 Bedrooms (corridors)

3 × UniFi U6 Lite

- Wi‑Fi 6

- Handles TVs & phones well

- Price: USD 99 eachui

### 🔹 Reception + Restaurant + Nightclub

3 × UniFi U6 LR

- Long‑range

- High client density

- Price: USD 179–199 each (average market price)

## ✅ Estimated Total Cost (Hardware Only)

| Item | Qty | Approx Total |
| --- | --- | --- |
| UDM‑SE | 1 | $499 |
| Lite 16 PoE Switch | 1 | $199 |
| U6 Lite | 3 | $297 |
| U6 LR | 3 | ~$550 |
| TOTAL |  | ~USD 1,545 |

📌 Excludes cabling & installation

# 4️⃣ Hotel Internet = 30 Mbps (Reality Check)

### ✅ With proper control:

✔ Web browsing ✅
✔ YouTube SD ✅
✔ WhatsApp / VoIP ✅
✔ Smart TVs ✅

### ❌ Without control:

❌ Buffering
❌ VoIP drops
❌ Complaints

✅ Bandwidth profiles are mandatory

# 5️⃣ YouTube Tutorials (Highly Recommended)

### 🎥 Beginner Setup

- UniFi Guest Portal & Vouchers
https://www.youtube.com/watch?v=HOXQIss3NIYyoutube

- UniFi Guest Network (New UI)
https://www.youtube.com/watch?v=lulaY4cOUzYyoutube

### 🎥 Hotel / Captive Portal

- Full Captive Portal Walkthrough
https://www.youtube.com/watch?v=1JYEbbFbpjoyoutube

- Voucher‑Based Access
https://www.youtube.com/watch?v=_IYwWRjr56gyoutube

# 6️⃣ How to Learn UniFi SAFELY (NO RISK TO CLIENT)

### ✅ Best Method (Professional Way)

#### 🔹 Step 1 — Mini Lab

Buy or borrow:

- 1 UniFi AP

- Use demo mode or a UDM

Practice: ✔ Create SSIDs
✔ Guest portal
✔ 1‑hour timeout
✔ Vouchers

#### 🔹 Step 2 — UniFi Simulator

Use UniFi Design Center

- Draw hotel layout

- Simulate AP placement

#### 🔹 Step 3 — Deploy in Phases

- Install hardware

- Test with 5 devices

- Test voucher expiry

- Test VoIP call

- THEN go live

# 7️⃣ Final Professional Advice ✅

You CAN safely deploy this even as a first‑timer IF: ✔ You use UDM‑SE
✔ You limit bandwidth
✔ You test vouchers
✔ You separate networks

👉 UniFi is forgiving — mistakes rarely break everything.

If you want next: ✅ Exact AP placement map ✅ Ready guest‑portal text (legal‑safe) ✅ VoIP‑optimized UniFi settings ✅ Client handover document

Do these:

✅ Exact AP placement map

✅ Ready guest‑portal text (legal‑safe)

✅ VoIP‑optimized UniFi settings

✅ Client handover document

And is there a possibility to practice on Unifi for free?

###### Copilot said:

Copilot

Excellent — let’s deliver everything you asked for, ready to use, and safe for a first UniFi deployment.
I’ll keep this practical, copy‑paste friendly, and hotel‑grade.

# ✅ 1) Exact AP Placement Map (18‑Room Hotel)

Because I don’t have architectural drawings, this is a field‑proven logical placement that works very well for hotels like yours.

## 🏨 Assumptions

- 18 bedrooms along corridors

- Concrete walls

- One main building

- Restaurant upstairs, reception + nightclub downstairs

## 📍 AP Placement Diagram (Text Map)

FIRST FLOOR (RESTAURANT + ROOMS)

--------------------------------

| RM | RM | RM | AP-1 | RM | RM | RM |

| RM | RM | RM |      | RM | RM | RM |

| RM | RM | RM | AP-2 | RM | RM | RM |

| RM | RM | RM |      | RM | RM | RM |

|-------------RESTAURANT-------------|

|              AP-6                  |

-------------------------------------

GROUND FLOOR

-------------------------------------

| RECEPTION |   LOBBY   | NIGHTCLUB |

|   AP-4    |   AP-3    |   AP-5    |

-------------------------------------

## ✅ Exact AP Models & Purpose

| AP | Model | Covers |
| --- | --- | --- |
| AP‑1 | U6 Lite | Rooms (first corridor area) |
| AP‑2 | U6 Lite | Rooms (second corridor area) |
| AP‑3 | U6 LR | Reception + Lobby |
| AP‑4 | U6 LR | Staff + check‑in area |
| AP‑5 | U6 LR | Nightclub (high density) |
| AP‑6 | U6 LR | Restaurant (paid Wi‑Fi) |

✅ Ceiling mount
✅ Never install APs inside rooms
✅ Keep ~8–10 m between APs

# ✅ 2) Ready, Legal‑Safe Guest Portal Text (Copy‑Paste)

Use this exact wording. It avoids legal problems and guest complaints.

## 🔐 Captive Portal – Welcome Text

Title

Welcome to [HOTEL NAME] Wi‑Fi

Main Message

This wireless internet service is provided for your convenience.
You are granted 1 hour of free internet access.

After Expiry Notice

After your free session expires, please contact reception or restaurant staff to continue using the service.

## 📜 Terms of Use (Legal‑Safe)

By accessing this Wi‑Fi network, you agree to the following terms:

• This service is provided "as is" without guarantees of speed or availability.

• Illegal activities, including downloading copyrighted material, are strictly prohibited.

• The hotel is not responsible for loss of data, security issues, or device damage.

• Network usage may be limited or suspended at any time.

By clicking “Connect”, you agree to these terms.

✅ Simple
✅ Hotel‑safe
✅ UniFi‑approved style

# ✅ 3) VoIP‑Optimized UniFi Settings (VERY IMPORTANT)

Your 30 Mbps internet can support VoIP ONLY if these are set.

## 🎧 UniFi Settings for VoIP Quality

### 🔹 A. Traffic & Bandwidth Control

Go to Settings → Profiles → User Groups

| Group | Download | Upload |
| --- | --- | --- |
| Guest‑Rooms | 3 Mbps | 1 Mbps |
| Restaurant | 5 Mbps | 2 Mbps |
| Staff | Unlimited | Unlimited |

✅ Prevents YouTube from killing WhatsApp calls

### 🔹 B. Wireless Settings (Global)

Settings → WiFi

✅ Enable:

- Band Steering

- Fast Roaming

- Proxy ARP

- Minimum RSSI (optional: ‑75 dBm)

❌ Disable:

- Legacy 802.11b rates

### 🔹 C. QoS / Smart Queues (UDM‑SE)

Settings → Internet → WAN → Smart Queues

- Enable Smart Queues

- Set:

  - Download: 28 Mbps

  - Upload: 26 Mbps

✅ This reserves bandwidth for VoIP
✅ Huge improvement in call quality

### 🔹 D. AP‑Level Tweaks

For Nightclub & Restaurant APs:

- Prefer 5 GHz

- Reduce 2.4 GHz power slightly

- Channel width: 40 MHz

# ✅ 4) Client Handover Document (You Can Print This)

Use this as your professional handover.

## 📄 HOTEL Wi‑Fi HANDOVER DOCUMENT

### Network Overview

- Wi‑Fi system: Ubiquiti UniFi

- Controller: UniFi Dream Machine SE

- Internet speed: 30 Mbps

### Wi‑Fi Networks

- Hotel‑Rooms

  - For guests in bedrooms

  - Supports phones & TVs

  - Speed optimized

- Restaurant‑WiFi

  - 1 hour free access

  - Paid access via vouchers after expiry

- Hotel‑Staff

  - Private internal network

### Guest Internet Policy

- One hour free per device

- Voucher required after expiry

- Bandwidth controlled to ensure fair usage

### Support & Maintenance

- System centrally managed

- Access points auto‑recover after power loss

- Password changes can be done by admin

### Admin Notes (Optional)

- Monthly password change recommended

- Router and switch connected to UPS

- Do not reset UniFi devices without admin approval

Installed by: __________________
Date: __________________

# ✅ 5) Can You Practice UniFi for FREE? YES ✅✅✅

This is VERY important for you.

## 🆓 Free & SAFE Ways to Learn UniFi

### ✅ Option 1: UniFi Network Application (Free)

- Download UniFi Controller

- Install on:

  - Laptop

  - Desktop

- Works without hardware

- Lets you:

  - Build networks

  - Create SSIDs

  - Practice guest portal

  - Learn the UI

✅ 100% free
✅ Perfect for beginners

### ✅ Option 2: UniFi OS Demo Mode

- Log in to UniFi Cloud

- Use Demo / Simulation

- Explore:

  - Hotspot

  - Vouchers

  - Users

  - Statistics

✅ No risk
✅ No hardware needed

### ✅ Option 3: One‑AP Home Lab (Best Method)

Buy:

- 1 × UniFi AP (U6 Lite)

Practice:

- Guest portal

- Voucher expiration

- Phone + TV testing

- VoIP calls

✅ This makes you confident before client deployment

# 🎯 Final Professional Advice (Very Important)

You are doing this the right way.

✅ UniFi is beginner‑friendly
✅ Your design is realistic
✅ 30 Mbps is usable with limits
✅ Hotel use‑case matches UniFi perfectly

If you want next, I can: ✅ Walk you click‑by‑click through the UniFi UI ✅ Create voucher pricing strategy ✅ Help you explain this clearly to the hotel owner ✅ Simulate “guest complaints” and how to fix them